ModelIt!

Student Activity Pack

How Do Doctors See Inside You Without Cutting You Open?

Modeling MRI Physics and Medical Imaging Technology

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Grade Level: 10th Grade — Level 2: Systems Under Pressure  |  Standard: HS-PS2-4, HS-PS4-4

What's Inside

| Name: _______________________________ | Date: _______________ |
| --- | --- |

1. Pre-Assessment  -  What do you already know?

2. Scoring Rubric  -  How your work will be assessed

3. Component Analysis  -  Sort and classify system parts

4. Model Recording Page  -  Sketch your model

5. Simulation Observations  -  Record what happens

6. Research & Extend  -  Add new components

7. Reflection Questions  -  What did you learn?

8. STEM Challenge  -  Design a Next-Generation Portable MRI System

Pre-Assessment: What Do You Know?

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer these questions BEFORE the lesson begins.

What do you know about how MRI machines create images of the inside of the body?

Why might a doctor choose MRI over an X-ray for certain types of injuries or conditions?

Draw a diagram showing what you think happens inside an MRI machine when it scans a patient.

What role do magnets play in medical imaging — how could a magnet help see inside a person?

Scoring Rubric

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Use this rubric to check your own work!

| Skill | 4 - Expert | 3 - Proficient | 2 - Developing | 1 - Beginning |
| --- | --- | --- | --- | --- |
| Identifying Components | All correct + explains role | Most correct | Some correct | Struggles to identify |
| Establishing Relationships | All arrows correct + explains | Most correct | Some correct | Incorrect |
| Running Simulations | Tests all; predicts outcomes | Runs all; records | Runs some with guidance | Needs help |
| Explaining Results | Clear with evidence | Good explanation | Partial | Struggles |

Component Analysis

| Name: _______________________________ | Date: _______________ |
| --- | --- |

What is a Component?

A component is a part of a system. In our How Do Doctors See Inside You Without Cutting You Open? system, we have these components:

*  Magnetic Field Strength: The intensity of the static magnetic field in Tesla, which aligns hydrogen nuclei in the patient's body and determines the signal-to-noise ratio of the resulting images

*  Coil Temperature: The temperature of the superconducting magnet coils, which must remain below the critical temperature (-269 degrees Celsius for NbTi) to maintain zero electrical resistance and stable magnetic field

*  Image Resolution: The spatial detail visible in the MRI image, measured in millimeters — determined by magnetic field strength, gradient coil precision, and signal-to-noise ratio

*  Tissue Contrast: The ability to visually distinguish different tissue types in the image based on their different relaxation times — the primary advantage of MRI over other imaging modalities

*  Patient Safety: The level of risk to the patient during scanning, affected by magnetic field interactions with implants, specific absorption rate (SAR) of radiofrequency energy, and acoustic noise from gradient switching

*  Energy Consumption: The total electrical power required to operate the MRI system, including maintaining cryogenic cooling for superconducting coils, powering gradient amplifiers, and running computing systems

*  Diagnostic Accuracy: The probability that the MRI image provides sufficient information for a correct medical diagnosis, determined by the combination of Image Resolution, Tissue Contrast, and the radiologist's expertise

Sorting Activity

Sort into EXTERNAL (inputs from outside) and INTERNAL (processes inside the system):

| EXTERNAL Components (Come from outside) | INTERNAL Components (Happen inside system) |
| --- | --- |

Explain your reasoning:

Model Recording Page

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Sketch your model showing all components and their relationships.

Relationship Key:

(+) POSITIVE: When one increases, the other ALSO increases

(-) NEGATIVE: When one increases, the other DECREASES

My Model Sketch:

Draw components as boxes. Connect with arrows labeled + or -.

Explain ONE relationship in your model. Why does it work that way?

Simulation Observations

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Run each scenario in ModelIt! and record your observations.

Standard 1.5T

Settings: Clinical standard field strength

What do you predict is the relationship between Magnetic Field Strength and Image Resolution at 1.5 Tesla?

Observation: What actually happened? Describe what you saw on the graph.

High-Field 3T

Settings: Double the standard field strength

What do you predict happens to Tissue Contrast and Patient Safety when Magnetic Field Strength doubles from 1.5T to 3T?

Observation: What actually happened? Describe what you saw on the graph.

Coil Failure

Settings: Superconducting coil warms above critical temperature

What do you predict happens to the entire MRI system if the liquid helium cooling fails and Coil Temperature rises?

Observation: What actually happened? Describe what you saw on the graph.

Research & Extend

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Real systems are more complex. EXTEND your model with new components!

Possible New Components:

*  Gradient Coil Precision: The accuracy of the additional magnetic field gradients that encode spatial location in the MRI signal — more precise gradients produce higher resolution images but generate louder noise and more heat

*  Scan Duration: The total time required to acquire sufficient data for a diagnostic-quality image — longer scans produce better images but increase patient discomfort, motion artifacts, and system operating costs

*  RF Pulse Sequence Design: The specific pattern and timing of radiofrequency pulses used to generate tissue-specific contrast — different sequences (T1-weighted, T2-weighted, FLAIR) highlight different tissue properties for different diagnostic purposes

My Extension:

New component I want to add: ________________________________

How does it connect? (Draw and explain)

What evidence supports these connections?

Reflection Questions

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer after completing all activities.

How does your model demonstrate why MRI is preferred over CT scans for soft tissue imaging despite being more expensive and slower?

What does the coil failure scenario reveal about the engineering fragility of the superconducting magnet system?

Why is there an optimal Magnetic Field Strength rather than simply using the strongest possible magnet?

How does your model help explain the extreme cost of MRI machines ($1-3 million) and MRI scans ($500-$5,000)?

What are the limitations of modeling MRI physics with only seven components when real MRI involves quantum spin physics, Fourier transforms, and complex pulse sequence programming?

STEM CHALLENGE: Design a Next-Generation Portable MRI System

| Name: _______________________________ | Date: _______________ |
| --- | --- |

YOUR MISSION

Design a compact, lower-cost MRI system that could be deployed in rural clinics, ambulances, or developing countries where traditional MRI is unavailable.

THE SCENARIO

A medical technology startup wants to bring MRI capabilities to the 70% of the world's population that currently has no access to MRI imaging. Your team has been hired to design a portable MRI system that sacrifices some image quality for dramatic reductions in cost, size, and power requirements.

ENGINEERING DESIGN THINKING

What type of magnet technology would you use — superconducting, permanent, or resistive electromagnets?

How would you power the system in locations without reliable electricity?

What imaging protocols would you prioritize for the most impactful diagnoses in resource-limited settings?

How would you address patient safety with a system that might be operated by non-specialist technicians?

MY DESIGN

Draw your design here:

EXPLAIN YOUR DESIGN

Why did you design it this way? Use scientific reasoning!