ModelIt!

Student Activity Pack

Carbon Capture at Scale: Can We Vacuum the Atmosphere?

Modeling the Engineering and Economics of Removing CO2 from the Sky

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Grade Level: 10th Grade — Level 3: Advanced Engineering & Design  |  Standard: HS-ESS2-6, HS-PS1-4

What's Inside

| Name: _______________________________ | Date: _______________ |
| --- | --- |

1. Pre-Assessment  -  What do you already know?

2. Scoring Rubric  -  How your work will be assessed

3. Component Analysis  -  Sort and classify system parts

4. Model Recording Page  -  Sketch your model

5. Simulation Observations  -  Record what happens

6. Research & Extend  -  Add new components

7. Reflection Questions  -  What did you learn?

8. STEM Challenge  -  Design a Carbon Capture Business Plan

Pre-Assessment: What Do You Know?

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer these questions BEFORE the lesson begins.

What do you think would happen if we could suck CO2 directly out of the atmosphere?

Why might removing CO2 from the air be harder than preventing it from being emitted in the first place?

Draw what you think a carbon capture system would look like and how it would work.

If carbon capture technology exists, why have we not solved climate change already?

Scoring Rubric

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Use this rubric to check your own work!

| Skill | 4 - Expert | 3 - Proficient | 2 - Developing | 1 - Beginning |
| --- | --- | --- | --- | --- |
| Identifying Components | All correct + explains role | Most correct | Some correct | Struggles to identify |
| Establishing Relationships | All arrows correct + explains | Most correct | Some correct | Incorrect |
| Running Simulations | Tests all; predicts outcomes | Runs all; records | Runs some with guidance | Needs help |
| Explaining Results | Clear with evidence | Good explanation | Partial | Struggles |

Component Analysis

| Name: _______________________________ | Date: _______________ |
| --- | --- |

What is a Component?

A component is a part of a system. In our Carbon Capture at Scale: Can We Vacuum the Atmosphere? system, we have these components:

*  CO2 Concentration: The atmospheric concentration of carbon dioxide at the capture site, currently approximately 420 parts per million and rising — higher concentrations make capture easier but indicate worsening climate conditions

*  Capture Rate: The mass of CO2 removed from the atmosphere per unit time, measured in tons per day — a single large facility currently captures about 4,000 tons per year, while global emissions exceed 36 billion tons per year

*  Sorbent Efficiency: The percentage of CO2 molecules that the sorbent material successfully captures from the air passing through the contactor — affected by temperature, humidity, sorbent degradation, and contact time

*  Energy Input: The total energy required to operate fans, regenerate sorbents, compress CO2, and run support systems — currently 6-10 gigajoules per ton of CO2 captured

*  Storage Capacity: The total volume of CO2 that can be permanently sequestered in available geological formations — globally estimated at 2,000+ gigatons, far exceeding removal needs

*  Reaction Temperature: The temperature required to release captured CO2 from the sorbent and regenerate it for reuse — typically 80-120 degrees Celsius for solid sorbents, requiring significant thermal energy

*  Regeneration Cost: The economic cost of the energy and materials needed to regenerate sorbents and compress CO2 for transport and storage — the single largest operating expense in direct air capture

*  Transport Distance: The distance between the capture facility and the geological storage site — longer distances increase cost and energy consumption for pipeline or tanker transport of compressed CO2

*  Geological Sequestration: The effectiveness and permanence of underground CO2 storage, including injection rate, formation integrity, monitoring requirements, and risk of leakage over centuries

*  Carbon Credit Value: The market price paid per ton of CO2 removed, which must exceed the total cost of capture, transport, and storage for the operation to be economically viable — currently ranging from $100-$1,000 per ton

Sorting Activity

Sort into EXTERNAL (inputs from outside) and INTERNAL (processes inside the system):

| EXTERNAL Components (Come from outside) | INTERNAL Components (Happen inside system) |
| --- | --- |

Explain your reasoning:

Model Recording Page

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Sketch your model showing all components and their relationships.

Relationship Key:

(+) POSITIVE: When one increases, the other ALSO increases

(-) NEGATIVE: When one increases, the other DECREASES

My Model Sketch:

Draw components as boxes. Connect with arrows labeled + or -.

Explain ONE relationship in your model. Why does it work that way?

Simulation Observations

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Run each scenario in ModelIt! and record your observations.

Current Technology

Settings: 2024-era capture technology at 420 ppm with fossil energy mix

What do you predict the net CO2 removal will be when the capture plant itself consumes fossil energy?

Observation: What actually happened? Describe what you saw on the graph.

Clean Energy Powered

Settings: All operations powered by renewable energy with zero emissions

What do you predict happens to the true climate impact when the energy source is completely clean?

Observation: What actually happened? Describe what you saw on the graph.

Scaled to Climate Impact

Settings: Operations scaled to 1 billion tons per year of capture

What do you predict are the barriers to scaling capture from 4,000 tons per year to 1 billion tons per year?

Observation: What actually happened? Describe what you saw on the graph.

Research & Extend

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Real systems are more complex. EXTEND your model with new components!

Possible New Components:

*  Sorbent Lifetime: The number of capture-regeneration cycles a sorbent material can undergo before degrading to the point of replacement — longer lifetimes reduce material costs and waste

*  Water Consumption: The volume of water used in capture processes and cooling systems — a significant concern in arid regions where water scarcity may limit facility siting

*  CO2 Utilization Revenue: Income from selling captured CO2 for industrial use — concrete curing, synthetic fuels, carbonated beverages — which can offset capture costs but does not permanently remove CO2 from the carbon cycle

My Extension:

New component I want to add: ________________________________

How does it connect? (Draw and explain)

What evidence supports these connections?

Reflection Questions

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer after completing all activities.

How does your model demonstrate the critical importance of the energy source in determining whether carbon capture is a net climate benefit?

What does the cost per ton comparison between direct air capture and other CO2 removal methods reveal about the role of each in climate strategy?

Why might carbon capture be necessary even if it is more expensive than simply reducing emissions?

How would your model change if carbon credit prices rose to $500 per ton globally?

What are the ethical implications of relying on carbon capture technology to address climate change versus reducing emissions at the source?

STEM CHALLENGE: Design a Carbon Capture Business Plan

| Name: _______________________________ | Date: _______________ |
| --- | --- |

YOUR MISSION

Design a commercially viable direct air capture facility that maximizes net CO2 removal while achieving economic sustainability through carbon credit sales.

THE SCENARIO

A climate technology venture fund is evaluating proposals for next-generation direct air capture facilities. Your team must design a facility that specifies the capture technology, energy source, geological storage location, and business model — demonstrating that the operation can remove CO2 profitably at a carbon credit price of $250 per ton within 10 years.

ENGINEERING DESIGN THINKING

What capture technology does your facility use and what is its sorbent efficiency?

How will you supply the thermal energy needed for sorbent regeneration without fossil fuels?

What geological formation will you use for storage and how will you ensure permanence?

What is your projected cost per ton at year 1 versus year 10, and what drives the cost reduction?

MY DESIGN

Draw your design here:

EXPLAIN YOUR DESIGN

Why did you design it this way? Use scientific reasoning!