ModelIt!

Student Activity Pack

Can the Ocean Power Entire Cities?

Modeling Ocean Thermal Energy Conversion and Marine Renewable Power

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Grade Level: 10th Grade — Level 3: Advanced Engineering & Design  |  Standard: HS-PS3-2, HS-ESS3-1

What's Inside

| Name: _______________________________ | Date: _______________ |
| --- | --- |

1. Pre-Assessment  -  What do you already know?

2. Scoring Rubric  -  How your work will be assessed

3. Component Analysis  -  Sort and classify system parts

4. Model Recording Page  -  Sketch your model

5. Simulation Observations  -  Record what happens

6. Research & Extend  -  Add new components

7. Reflection Questions  -  What did you learn?

8. STEM Challenge  -  Design an OTEC-Powered Island Energy System

Pre-Assessment: What Do You Know?

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer these questions BEFORE the lesson begins.

How do you think the temperature of the ocean changes as you go deeper?

Could you use a temperature difference to generate electricity? How might that work?

Draw what you think a power plant that runs on warm and cold water would look like.

Why do you think ocean energy is not widely used if the ocean covers 70% of the Earth?

Scoring Rubric

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Use this rubric to check your own work!

| Skill | 4 - Expert | 3 - Proficient | 2 - Developing | 1 - Beginning |
| --- | --- | --- | --- | --- |
| Identifying Components | All correct + explains role | Most correct | Some correct | Struggles to identify |
| Establishing Relationships | All arrows correct + explains | Most correct | Some correct | Incorrect |
| Running Simulations | Tests all; predicts outcomes | Runs all; records | Runs some with guidance | Needs help |
| Explaining Results | Clear with evidence | Good explanation | Partial | Struggles |

Component Analysis

| Name: _______________________________ | Date: _______________ |
| --- | --- |

What is a Component?

A component is a part of a system. In our Can the Ocean Power Entire Cities? system, we have these components:

*  Surface Water Temperature: The temperature of seawater at the ocean surface, typically 25-30 degrees Celsius in tropical regions — the heat source for the OTEC system that evaporates the working fluid

*  Deep Water Temperature: The temperature of seawater pumped from 800-1,000 meters depth, typically 4-5 degrees Celsius — the cold sink that condenses the working fluid after it passes through the turbine

*  Temperature Differential: The difference between surface and deep water temperatures — at least 20 degrees Celsius is needed for viable OTEC operation, and efficiency increases with larger differentials

*  Fluid Flow Rate: The volume of seawater pumped through the heat exchangers per second — both warm surface water and cold deep water must flow at enormous rates to extract sufficient thermal energy

*  Turbine Efficiency: The percentage of thermal energy in the working fluid vapor that is converted to mechanical rotation of the turbine — limited by practical engineering to 80-90% of the theoretical Carnot maximum

*  Energy Conversion: The net electrical energy output after subtracting the energy consumed by massive seawater pumps, working fluid pumps, and auxiliary systems — typically only 2-3% of the total thermal energy processed

*  Environmental Impact: The ecological effects of pumping billions of gallons of deep cold water to the surface, including nutrient upwelling, temperature changes in surrounding waters, and disruption of marine ecosystems

*  Infrastructure Cost: The capital and operating cost of the massive offshore platform, deep-water intake pipes (1,000+ meters long), heat exchangers, turbines, and submarine power cables — currently $25,000-$50,000 per installed kilowatt

*  Power Output: The total electricity delivered to shore in megawatts — current designs target 10-100 MW per platform, compared to 1,000+ MW for conventional power plants

Sorting Activity

Sort into EXTERNAL (inputs from outside) and INTERNAL (processes inside the system):

| EXTERNAL Components (Come from outside) | INTERNAL Components (Happen inside system) |
| --- | --- |

Explain your reasoning:

Model Recording Page

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Sketch your model showing all components and their relationships.

Relationship Key:

(+) POSITIVE: When one increases, the other ALSO increases

(-) NEGATIVE: When one increases, the other DECREASES

My Model Sketch:

Draw components as boxes. Connect with arrows labeled + or -.

Explain ONE relationship in your model. Why does it work that way?

Simulation Observations

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Run each scenario in ModelIt! and record your observations.

Tropical Optimal

Settings: 28 degrees Celsius surface, 4 degrees Celsius deep, 24 degree differential

What do you predict the maximum Energy Conversion efficiency and Power Output will be with optimal tropical conditions?

Observation: What actually happened? Describe what you saw on the graph.

Reduced Differential

Settings: Temperature differential drops to 15 degrees Celsius

What do you predict happens to Power Output when the temperature difference shrinks by just 5 degrees?

Observation: What actually happened? Describe what you saw on the graph.

Maximum Scale

Settings: Fluid flow rate increased to maximum platform capacity

What do you predict are the limits to scaling up — can you just pump more water to get more power?

Observation: What actually happened? Describe what you saw on the graph.

Research & Extend

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Real systems are more complex. EXTEND your model with new components!

Possible New Components:

*  Desalinated Water Output: OTEC systems can produce fresh water as a byproduct by condensing evaporated seawater — a valuable co-product for island communities with limited freshwater resources

*  Cold Water Agriculture: The cold deep water discharge can be used for air conditioning and cold-water agriculture (growing temperate crops in tropical climates), improving the economic viability of the OTEC system

*  Biofouling Rate: The rate at which marine organisms (barnacles, algae, biofilm) colonize heat exchanger surfaces, reducing thermal transfer efficiency and requiring periodic cleaning or anti-fouling treatment

My Extension:

New component I want to add: ________________________________

How does it connect? (Draw and explain)

What evidence supports these connections?

Reflection Questions

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer after completing all activities.

How does the Carnot efficiency limit fundamentally constrain OTEC performance, and why is this different from solar or wind energy constraints?

What does your model reveal about the relationship between temperature differential and economic viability?

Why is the 24/7 baseload nature of OTEC valuable even though its per-kWh cost is higher than solar or wind?

How would climate change (warming surface waters, potentially changing deep water temperatures) affect OTEC performance over the next century?

What are the ethical considerations of potentially disrupting marine ecosystems to generate clean energy?

STEM CHALLENGE: Design an OTEC-Powered Island Energy System

| Name: _______________________________ | Date: _______________ |
| --- | --- |

YOUR MISSION

Design a comprehensive OTEC-based energy system for a tropical island nation that currently depends entirely on imported diesel fuel for electricity.

THE SCENARIO

A small Pacific island nation of 50,000 people currently generates all electricity from imported diesel at $0.40 per kWh — the highest electricity cost in the world. They use approximately 30 MW peak power. Your team must design an OTEC system that can replace diesel generation, analyzing the thermodynamic, economic, and environmental feasibility of the project.

ENGINEERING DESIGN THINKING

Where would you site the platform relative to the island, and how deep must the cold water pipe extend?

What working fluid would you use and what is its thermodynamic cycle efficiency?

How would you manage the cold deep water discharge to minimize environmental disruption while potentially creating beneficial side products?

What is your projected cost per kilowatt-hour and payback period compared to continued diesel generation?

MY DESIGN

Draw your design here:

EXPLAIN YOUR DESIGN

Why did you design it this way? Use scientific reasoning!