ModelIt!

Student Activity Pack

Building a Space Elevator: The Ultimate Engineering Challenge

Modeling the Physics and Materials Science of a Tether to the Stars

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Grade Level: 10th Grade — Level 3: Advanced Engineering & Design  |  Standard: HS-PS2-2, HS-PS1-3

What's Inside

| Name: _______________________________ | Date: _______________ |
| --- | --- |

1. Pre-Assessment  -  What do you already know?

2. Scoring Rubric  -  How your work will be assessed

3. Component Analysis  -  Sort and classify system parts

4. Model Recording Page  -  Sketch your model

5. Simulation Observations  -  Record what happens

6. Research & Extend  -  Add new components

7. Reflection Questions  -  What did you learn?

8. STEM Challenge  -  Design a Space Elevator Materials Research Roadmap

Pre-Assessment: What Do You Know?

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer these questions BEFORE the lesson begins.

What do you think a space elevator would look like and how would it work?

Why do you think rockets are so expensive for reaching orbit, and could a cable be cheaper?

Draw the forces acting on a cable stretching from Earth's surface to space — what keeps it from falling down?

What material properties would a space elevator cable need and do any current materials qualify?

Scoring Rubric

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Use this rubric to check your own work!

| Skill | 4 - Expert | 3 - Proficient | 2 - Developing | 1 - Beginning |
| --- | --- | --- | --- | --- |
| Identifying Components | All correct + explains role | Most correct | Some correct | Struggles to identify |
| Establishing Relationships | All arrows correct + explains | Most correct | Some correct | Incorrect |
| Running Simulations | Tests all; predicts outcomes | Runs all; records | Runs some with guidance | Needs help |
| Explaining Results | Clear with evidence | Good explanation | Partial | Struggles |

Component Analysis

| Name: _______________________________ | Date: _______________ |
| --- | --- |

What is a Component?

A component is a part of a system. In our Building a Space Elevator: The Ultimate Engineering Challenge system, we have these components:

*  Cable Tensile Strength: The maximum stress the cable material can withstand before breaking — must exceed approximately 65 GPa for a tapered carbon nanotube cable design, compared to the current bulk manufacturing limit of approximately 1-2 GPa for carbon nanotube fibers

*  Gravitational Force Gradient: The variation in gravitational pull along the cable's length — strongest at the surface (9.8 m/s^2) and decreasing with altitude squared, creating a non-uniform load distribution that makes the cable thickest at geostationary orbit

*  Centrifugal Counterweight: The mass and position of the counterweight beyond geostationary orbit that provides the centrifugal tension to keep the cable taut — heavier counterweights at greater distances reduce stress in the lower cable sections

*  Climber Mass: The mass of the vehicle that travels up and down the cable carrying payload — heavier climbers stress the cable more but can carry more cargo, creating a direct trade-off between capability and safety margin

*  Orbital Altitude: The height at which the cable's center of mass must be positioned — at or above 35,786 km for geostationary equilibrium — with the total cable extending beyond this to accommodate the counterweight

*  Material Limits: The gap between theoretical material properties (what physics allows) and manufacturable properties (what we can actually make) — carbon nanotubes theoretically meet requirements but cannot yet be manufactured at the needed scale or quality

*  Energy Consumption: The total energy required for a climber to ascend from the surface to geostationary orbit — approximately 50 MJ per kg, which must be transmitted to the climber via laser, microwave, or onboard power

*  Payload Capacity: The maximum mass that can be delivered to orbit per climber trip, determined by cable strength margin, climber design, and power system — the economic viability depends on sufficiently high payload per trip at low cost

*  Atmospheric Stress: The aerodynamic forces, weather impacts (lightning, wind, ice), and atmospheric chemical exposure on the lower 100 km of cable — this section faces challenges no other part of the structure encounters

*  Economic Viability: The cost per kilogram to orbit via space elevator compared to rocket launch — the elevator must deliver cargo below $200/kg to be economically compelling, compared to current rocket costs of $2,000-10,000/kg

Sorting Activity

Sort into EXTERNAL (inputs from outside) and INTERNAL (processes inside the system):

| EXTERNAL Components (Come from outside) | INTERNAL Components (Happen inside system) |
| --- | --- |

Explain your reasoning:

Model Recording Page

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Sketch your model showing all components and their relationships.

Relationship Key:

(+) POSITIVE: When one increases, the other ALSO increases

(-) NEGATIVE: When one increases, the other DECREASES

My Model Sketch:

Draw components as boxes. Connect with arrows labeled + or -.

Explain ONE relationship in your model. Why does it work that way?

Simulation Observations

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Run each scenario in ModelIt! and record your observations.

Carbon Nanotube Ideal

Settings: Theoretical maximum tensile strength of perfect carbon nanotubes

What do you predict happens to system feasibility if we could manufacture carbon nanotubes at their theoretical strength in 100,000 km lengths?

Observation: What actually happened? Describe what you saw on the graph.

Current Materials

Settings: Best currently manufacturable cable material

What do you predict happens when you try to build a space elevator with the strongest material we can actually make today?

Observation: What actually happened? Describe what you saw on the graph.

Climber Stress Test

Settings: Maximum economic payload capacity

What do you predict is the relationship between payload size and the Cable Tensile Strength required?

Observation: What actually happened? Describe what you saw on the graph.

Research & Extend

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Real systems are more complex. EXTEND your model with new components!

Possible New Components:

*  Orbital Debris Risk: The probability of the cable being struck by space debris or micrometeorites at various altitudes — the cable crosses all orbital zones and must either avoid or survive millions of potential impacts over its operational lifetime

*  Climber Power System: The method of transmitting energy to the ascending climber — ground-based lasers, microwave beaming, or onboard generators — each with different efficiency, weight, and safety trade-offs

*  Anchor Station Design: The engineering of the ground-level platform, likely a mobile ocean platform near the equator, that must maintain cable tension, handle climber launch and recovery, and survive weather extremes

My Extension:

New component I want to add: ________________________________

How does it connect? (Draw and explain)

What evidence supports these connections?

Reflection Questions

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer after completing all activities.

How does your model demonstrate that the space elevator is a materials science problem rather than a physics or engineering problem?

What does the strength-to-weight ratio gap reveal about the relationship between theoretical material properties and manufacturing capability?

If the materials challenge were solved tomorrow, what other variables in your model would become the new limiting factors?

How would a functioning space elevator change the economics and accessibility of space — and what societal changes might follow?

What are the risks of a space elevator failure — what happens if a 100,000 km cable breaks?

STEM CHALLENGE: Design a Space Elevator Materials Research Roadmap

| Name: _______________________________ | Date: _______________ |
| --- | --- |

YOUR MISSION

Design a 25-year materials science research roadmap that identifies the specific breakthroughs needed to close the gap between current carbon nanotube manufacturing capability and space elevator requirements.

THE SCENARIO

A consortium of space agencies and technology companies has committed $10 billion over 25 years to develop the materials needed for a space elevator. Your team must design the research roadmap: identifying the specific manufacturing challenges, the intermediate milestones, the testing protocols, and the decision points where the program should continue, pivot, or redirect to alternative materials.

ENGINEERING DESIGN THINKING

What is the current state of carbon nanotube fiber manufacturing and what specific defects limit strength?

What manufacturing processes show the most promise for scaling nanotube quality and length simultaneously?

What intermediate applications (bridges, aircraft, body armor) could demonstrate progress and provide revenue during the 25-year development?

What alternative materials (boron nitride nanotubes, graphene ribbons, diamond nanothreads) should receive parallel research investment?

MY DESIGN

Draw your design here:

EXPLAIN YOUR DESIGN

Why did you design it this way? Use scientific reasoning!