ModelIt!

Student Activity Pack

How Do Noise-Canceling Headphones Work?

Modeling Wave Superposition and Destructive Interference

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Grade Level: 11th Grade — Level 1: Science & Society  |  Standard: HS-PS4-1, HS-PS4-5

What's Inside

| Name: _______________________________ | Date: _______________ |
| --- | --- |

1. Pre-Assessment  -  What do you already know?

2. Scoring Rubric  -  How your work will be assessed

3. Component Analysis  -  Sort and classify system parts

4. Model Recording Page  -  Sketch your model

5. Simulation Observations  -  Record what happens

6. Research & Extend  -  Add new components

7. Reflection Questions  -  What did you learn?

8. STEM Challenge  -  Design a Next-Generation Noise Cancellation System

Pre-Assessment: What Do You Know?

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer these questions BEFORE the lesson begins.

How do you think noise-canceling headphones work — do they block sound, absorb it, or do something else entirely?

What happens when two waves meet in the same place at the same time? Can you think of examples from water or sound?

Why do you think noise-canceling headphones are better at canceling airplane engine noise than they are at canceling human voices?

If I told you that noise-canceling headphones work by adding MORE sound to your ears, would you believe me? How could more sound make things quieter?

Scoring Rubric

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Use this rubric to check your own work!

| Skill | 4 - Expert | 3 - Proficient | 2 - Developing | 1 - Beginning |
| --- | --- | --- | --- | --- |
| Identifying Components | All correct + explains role | Most correct | Some correct | Struggles to identify |
| Establishing Relationships | All arrows correct + explains | Most correct | Some correct | Incorrect |
| Running Simulations | Tests all; predicts outcomes | Runs all; records | Runs some with guidance | Needs help |
| Explaining Results | Clear with evidence | Good explanation | Partial | Struggles |

Component Analysis

| Name: _______________________________ | Date: _______________ |
| --- | --- |

What is a Component?

A component is a part of a system. In our How Do Noise-Canceling Headphones Work? system, we have these components:

*  Ambient Noise Frequency: The frequency of the environmental sound to be canceled, measured in Hertz — low-frequency rumbles (airplane engines at 80-200 Hz, traffic at 100-500 Hz) are easier to cancel than high-frequency sounds (conversation at 300-3,000 Hz, birdsong at 2,000-8,000 Hz)

*  Ambient Noise Amplitude: The loudness of the environmental noise, measured in decibels — louder noise requires a stronger anti-phase signal for complete cancellation

*  Processing Speed: How quickly the headphone's microphone-processor-speaker system can sample incoming noise, compute the anti-phase signal, and output it — measured in microseconds. Faster processing enables cancellation of higher-frequency sounds

*  Phase Accuracy: How precisely the anti-phase signal aligns with the incoming noise wave — even a small phase error reduces cancellation effectiveness. Perfect cancellation requires exact 180-degree phase offset at every frequency

*  Cancellation Effectiveness: The percentage of ambient noise amplitude that is eliminated by destructive interference, measured as noise reduction in decibels — top-tier headphones achieve 20-30 dB reduction at low frequencies

Sorting Activity

Sort into EXTERNAL (inputs from outside) and INTERNAL (processes inside the system):

| EXTERNAL Components (Come from outside) | INTERNAL Components (Happen inside system) |
| --- | --- |

Explain your reasoning:

Model Recording Page

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Sketch your model showing all components and their relationships.

Relationship Key:

(+) POSITIVE: When one increases, the other ALSO increases

(-) NEGATIVE: When one increases, the other DECREASES

My Model Sketch:

Draw components as boxes. Connect with arrows labeled + or -.

Explain ONE relationship in your model. Why does it work that way?

Simulation Observations

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Run each scenario in ModelIt! and record your observations.

Low-Frequency Cancellation

Settings: Noise: 100 Hz sine wave at 80 dB | ANC: Active

How many decibels of noise reduction do you predict at 100 Hz? Will the cancellation be nearly perfect or partial?

Observation: What actually happened? Describe what you saw on the graph.

High-Frequency Challenge

Settings: Noise: 4,000 Hz sine wave at 70 dB | ANC: Active

Do you predict noise cancellation will be as effective at 4,000 Hz as at 100 Hz? Why or why not?

Observation: What actually happened? Describe what you saw on the graph.

Mixed Environment

Settings: Noise: Airplane cabin spectrum (multiple frequencies) | ANC: Active

In a real airplane cabin with noise at many frequencies simultaneously, which frequency components do you predict will be canceled most effectively?

Observation: What actually happened? Describe what you saw on the graph.

Research & Extend

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Real systems are more complex. EXTEND your model with new components!

Possible New Components:

*  Feedback vs. Feedforward Architecture: Two approaches to ANC microphone placement: feedforward places the mic outside the ear cup (sensing noise before it arrives) while feedback places it inside (sensing the combined result). Each has trade-offs in latency, accuracy, and frequency range

*  Ear Seal Quality: The physical fit and acoustic seal between the headphone ear cup and the wearer's head — passive isolation from the physical seal blocks 10-25 dB of high-frequency noise that active cancellation struggles with, making passive and active noise reduction complementary

*  Adaptive Algorithm: Machine learning algorithms that learn the acoustic characteristics of common noise environments (airplane, train, office) and pre-load optimized anti-phase profiles, reducing the processing time needed for real-time cancellation

My Extension:

New component I want to add: ________________________________

How does it connect? (Draw and explain)

What evidence supports these connections?

Reflection Questions

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer after completing all activities.

How does your model demonstrate that noise cancellation is based on wave mathematics (superposition) rather than physical sound blocking?

Why does the same timing error that barely affects low-frequency cancellation completely ruin high-frequency cancellation?

What does the frequency-dependent effectiveness of noise cancellation reveal about the fundamental relationship between wavelength and precision?

How could the principles of destructive interference be applied beyond headphones — in architecture, vehicles, or industrial noise control?

What limitations does your model have in capturing the full complexity of real-world noise cancellation environments?

STEM CHALLENGE: Design a Next-Generation Noise Cancellation System

| Name: _______________________________ | Date: _______________ |
| --- | --- |

YOUR MISSION

Design an improved active noise cancellation system that extends effective cancellation to higher frequency ranges than current technology allows.

THE SCENARIO

A headphone manufacturer wants to develop the next generation of noise-canceling headphones that can cancel not just engine rumble but also human speech and other mid-frequency noise. Current technology achieves 25 dB reduction below 500 Hz but only 5 dB reduction above 2,000 Hz. Your acoustic engineering team must design a system architecture that improves high-frequency cancellation using faster processing, predictive algorithms, or multi-microphone arrays.

ENGINEERING DESIGN THINKING

What microphone placement strategy would minimize the distance sound must travel before being sampled, reducing processing delay?

How could multiple microphones create a spatial noise map that predicts incoming sound direction and characteristics?

What role could machine learning play in pre-computing anti-phase signals for common noise environments (airplane, subway, office)?

How would you test your improved system, and what metrics would prove it outperforms current technology?

MY DESIGN

Draw your design here:

EXPLAIN YOUR DESIGN

Why did you design it this way? Use scientific reasoning!