ModelIt!

Student Activity Pack

The Heat Island Effect

Modeling Urban Temperature Amplification, Health Impacts, and Green Infrastructure Solutions

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Grade Level: 11th Grade — Level 2: Complex Systems & Sustainability  |  Standard: HS-ESS3-4, HS-ETS1-1

What's Inside

| Name: _______________________________ | Date: _______________ |
| --- | --- |

1. Pre-Assessment  -  What do you already know?

2. Scoring Rubric  -  How your work will be assessed

3. Component Analysis  -  Sort and classify system parts

4. Model Recording Page  -  Sketch your model

5. Simulation Observations  -  Record what happens

6. Research & Extend  -  Add new components

7. Reflection Questions  -  What did you learn?

8. STEM Challenge  -  Design an Urban Heat Equity Action Plan

Pre-Assessment: What Do You Know?

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer these questions BEFORE the lesson begins.

Why do you think a parking lot feels hotter than a park on a summer day — what's actually different about those two surfaces?

Have you ever noticed temperature differences between different neighborhoods in your city? What do you think causes those differences?

Draw a diagram showing how you think a city's buildings and roads affect temperature compared to a forest or farm.

Who do you think is most affected by extreme heat in a city — and why might some neighborhoods be hotter than others?

Scoring Rubric

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Use this rubric to check your own work!

| Skill | 4 - Expert | 3 - Proficient | 2 - Developing | 1 - Beginning |
| --- | --- | --- | --- | --- |
| Identifying Components | All correct + explains role | Most correct | Some correct | Struggles to identify |
| Establishing Relationships | All arrows correct + explains | Most correct | Some correct | Incorrect |
| Running Simulations | Tests all; predicts outcomes | Runs all; records | Runs some with guidance | Needs help |
| Explaining Results | Clear with evidence | Good explanation | Partial | Struggles |

Component Analysis

| Name: _______________________________ | Date: _______________ |
| --- | --- |

What is a Component?

A component is a part of a system. In our The Heat Island Effect system, we have these components:

*  Urban Land Cover Composition: The proportion of a city's surface covered by impervious materials (buildings, roads, parking lots) versus vegetation and water features — the single most influential factor in heat island intensity, with heavily paved areas absorbing 80-95% of incoming solar radiation

*  Surface Heat Absorption Rate: The rate at which urban surfaces convert incoming solar radiation into stored thermal energy, determined by surface albedo, color, and material properties — dark asphalt can reach surface temperatures of 150°F (65°C) on summer days, while a grass surface under identical conditions stays below 95°F (35°C)

*  Waste Heat Generation: The anthropogenic heat released by vehicles, air conditioning systems, industrial processes, and building energy use — in dense urban cores, waste heat can add 2-4°F to ambient temperatures, and air conditioning creates a vicious cycle: more cooling inside means more waste heat outside

*  Evapotranspiration Cooling Rate: The rate of natural cooling provided by vegetation and water features through evapotranspiration — each mature tree provides cooling equivalent to 2-5 window air conditioning units running 20 hours per day, and urban parks can be 5-10°F cooler than surrounding built areas

*  Ambient Air Temperature: The actual air temperature experienced by people in different parts of the city, resulting from the balance between heat absorption, waste heat generation, and evapotranspiration cooling — can vary by 10-15°F between the hottest and coolest neighborhoods in the same city on the same day

*  Heat-Related Health Impact: The burden of heat illness, hospitalization, and mortality experienced by urban populations, which increases nonlinearly above heat thresholds — heat is the deadliest weather phenomenon in the United States, killing more people annually than hurricanes, floods, and tornadoes combined, with impacts concentrated in low-income neighborhoods with the least tree cover and the most impervious surface

Sorting Activity

Sort into EXTERNAL (inputs from outside) and INTERNAL (processes inside the system):

| EXTERNAL Components (Come from outside) | INTERNAL Components (Happen inside system) |
| --- | --- |

Explain your reasoning:

Model Recording Page

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Sketch your model showing all components and their relationships.

Relationship Key:

(+) POSITIVE: When one increases, the other ALSO increases

(-) NEGATIVE: When one increases, the other DECREASES

My Model Sketch:

Draw components as boxes. Connect with arrows labeled + or -.

Explain ONE relationship in your model. Why does it work that way?

Simulation Observations

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Run each scenario in ModelIt! and record your observations.

Current Urban Baseline

Settings: 60-80% impervious surface, minimal tree canopy in underserved areas

What do you predict is the temperature difference between the highest-canopy neighborhood and the lowest-canopy neighborhood on a 100°F day, and how does this affect Heat-Related Health Impact?

Observation: What actually happened? Describe what you saw on the graph.

Climate Change Amplification

Settings: Current heat island plus 3-5°F baseline warming

How many additional days per year do you predict will exceed dangerous heat thresholds (105°F+) in the most vulnerable urban neighborhoods when climate change compounds the heat island effect?

Observation: What actually happened? Describe what you saw on the graph.

Green Infrastructure Intervention

Settings: 30% tree canopy, 50% cool roofs, connected green corridors

Do you predict that green infrastructure can reduce heat island intensity enough to prevent the most dangerous climate-amplified heat scenarios, or are there limits to what vegetation and reflective surfaces can achieve?

Observation: What actually happened? Describe what you saw on the graph.

Research & Extend

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Real systems are more complex. EXTEND your model with new components!

Possible New Components:

*  Air Quality Degradation: Higher urban temperatures accelerate the chemical formation of ground-level ozone (smog) and increase particulate matter suspension, creating a heat-air quality feedback loop where the hottest neighborhoods also have the worst air quality — compounding respiratory health impacts

*  Energy Demand Feedback: Each 1°F increase in urban temperature increases peak electricity demand by 1.5-2% for air conditioning, stressing power grids, increasing fossil fuel combustion at power plants, and generating more waste heat — a positive feedback loop where cooling demand amplifies the heat that drives it

*  Stormwater Runoff Intensity: Impervious urban surfaces that cause heat islands also prevent water infiltration, increasing stormwater runoff volume and velocity during rain events — green infrastructure that addresses heat (permeable pavement, rain gardens, tree canopy) simultaneously reduces flood risk, creating co-benefits

My Extension:

New component I want to add: ________________________________

How does it connect? (Draw and explain)

What evidence supports these connections?

Reflection Questions

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer after completing all activities.

Why does your model show that the urban heat island is not just a climate problem but an environmental justice problem — and how do historical decisions about where to invest in green space still determine who suffers most from heat today?

How does the air conditioning feedback loop in your model reveal a paradox where individual cooling solutions make the collective problem worse?

What does your model reveal about why nighttime heat is more dangerous than daytime heat for human health, and how urban materials prevent nighttime cooling?

How do the co-benefits of green infrastructure — cooling, air quality, stormwater, mental health, property value — change the cost-benefit calculation compared to analyzing heat reduction alone?

What does your model suggest about the tension between designing cooling solutions for the hottest neighborhoods and the risk that those improvements trigger gentrification and displace the people they were meant to protect?

STEM CHALLENGE: Design an Urban Heat Equity Action Plan

| Name: _______________________________ | Date: _______________ |
| --- | --- |

YOUR MISSION

Design a neighborhood-scale cooling strategy that reduces peak summer temperatures by at least 8°F in the hottest, most vulnerable census tract of a major city within 15 years, using a combination of green infrastructure, cool surfaces, and community design.

THE SCENARIO

A city health department has identified a census tract where heat-related emergency room visits are 4x the city average. The neighborhood has 5% tree canopy (versus 25% citywide), 85% impervious surface cover, no parks within walking distance, and a population that is 70% low-income with high rates of outdoor workers and elderly residents without air conditioning. Your team has been funded to design a comprehensive cooling plan.

ENGINEERING DESIGN THINKING

What temperature monitoring data would you need to map heat exposure at the block level and identify the hottest microclimates within the neighborhood?

How would you prioritize cooling investments — should you start with the hottest blocks, the blocks with the most vulnerable residents, or the blocks where cooling infrastructure is most cost-effective?

What combination of short-term solutions (shade structures, cool roofs, misting stations) and long-term solutions (tree planting, park creation, green corridors) would your plan include?

How would you engage community residents in the design process to ensure the plan reflects their priorities and avoids unintended displacement?

MY DESIGN

Draw your design here:

EXPLAIN YOUR DESIGN

Why did you design it this way? Use scientific reasoning!