ModelIt!

Student Activity Pack

Can We Build a Fusion Reactor?

Modeling Plasma Confinement, Energy Balance, and the Path to Unlimited Clean Energy

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Grade Level: 11th Grade — Level 3: Emerging Science & Technology  |  Standard: HS-PS1-8, HS-PS3-2

What's Inside

| Name: _______________________________ | Date: _______________ |
| --- | --- |

1. Pre-Assessment  -  What do you already know?

2. Scoring Rubric  -  How your work will be assessed

3. Component Analysis  -  Sort and classify system parts

4. Model Recording Page  -  Sketch your model

5. Simulation Observations  -  Record what happens

6. Research & Extend  -  Add new components

7. Reflection Questions  -  What did you learn?

8. STEM Challenge  -  Design a Next-Generation Fusion Reactor Approach

Pre-Assessment: What Do You Know?

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer these questions BEFORE the lesson begins.

What is nuclear fusion and how does it differ from the nuclear fission used in today's power plants?

Why do you think it has taken over 70 years of research and billions of dollars without achieving commercial fusion power?

Draw a diagram of what you think happens inside a fusion reactor — what goes in, what comes out, and what keeps it working.

If fusion is the energy source of every star, what makes it so difficult to reproduce in a building on Earth?

Scoring Rubric

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Use this rubric to check your own work!

| Skill | 4 - Expert | 3 - Proficient | 2 - Developing | 1 - Beginning |
| --- | --- | --- | --- | --- |
| Identifying Components | All correct + explains role | Most correct | Some correct | Struggles to identify |
| Establishing Relationships | All arrows correct + explains | Most correct | Some correct | Incorrect |
| Running Simulations | Tests all; predicts outcomes | Runs all; records | Runs some with guidance | Needs help |
| Explaining Results | Clear with evidence | Good explanation | Partial | Struggles |

Component Analysis

| Name: _______________________________ | Date: _______________ |
| --- | --- |

What is a Component?

A component is a part of a system. In our Can We Build a Fusion Reactor? system, we have these components:

*  Plasma Temperature: The core temperature of the fusion fuel, which must exceed 150 million degrees Celsius for the deuterium-tritium reaction — at this temperature, nuclei have enough kinetic energy to overcome their electromagnetic repulsion and fuse

*  Magnetic Field Strength: The intensity of the superconducting magnets confining the plasma, measured in Tesla — stronger fields compress plasma to higher density and resist instabilities, but require enormous superconducting coils cooled to near absolute zero

*  Fuel Density: The concentration of deuterium and tritium nuclei in the plasma — higher density increases collision frequency and fusion reaction rate but also increases the power needed to sustain and confine the plasma

*  Energy Output: The total thermal energy produced by fusion reactions per second — determined by the product of plasma temperature, fuel density, and the fusion cross-section, which peaks at around 150 million degrees

*  Neutron Flux: The number of high-energy neutrons produced per unit area per second — each deuterium-tritium fusion reaction produces a 14.1 MeV neutron that carries 80% of the fusion energy and bombards reactor structural materials

*  Wall Material Degradation: The cumulative structural damage to the reactor first wall and blanket caused by decades of neutron bombardment — neutrons displace atoms from crystal lattices, create helium bubbles, and transmute structural elements into different atoms

*  Net Energy Gain: The Q factor representing whether the reactor produces net energy — determined by fusion energy output minus all energy inputs including plasma heating, magnet cooling, fuel injection, and facility operations

Sorting Activity

Sort into EXTERNAL (inputs from outside) and INTERNAL (processes inside the system):

| EXTERNAL Components (Come from outside) | INTERNAL Components (Happen inside system) |
| --- | --- |

Explain your reasoning:

Model Recording Page

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Sketch your model showing all components and their relationships.

Relationship Key:

(+) POSITIVE: When one increases, the other ALSO increases

(-) NEGATIVE: When one increases, the other DECREASES

My Model Sketch:

Draw components as boxes. Connect with arrows labeled + or -.

Explain ONE relationship in your model. Why does it work that way?

Simulation Observations

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Run each scenario in ModelIt! and record your observations.

ITER Design Parameters

Settings: 150 million degrees, 5.3 Tesla, moderate fuel density

What do you predict the Net Energy Gain (Q factor) will be under the conditions ITER was designed to achieve?

Observation: What actually happened? Describe what you saw on the graph.

Push to Ignition

Settings: Maximum achievable temperature, field strength, and density

What do you predict happens to both Energy Output and Wall Material Degradation when you push all inputs to their engineering limits?

Observation: What actually happened? Describe what you saw on the graph.

Confinement Collapse

Settings: Magnetic field strength reduced by 25%

What do you predict happens to the plasma state and fusion reactions when the magnetic bottle weakens significantly?

Observation: What actually happened? Describe what you saw on the graph.

Research & Extend

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Real systems are more complex. EXTEND your model with new components!

Possible New Components:

*  Tritium Breeding Ratio: The rate at which the lithium blanket surrounding the plasma produces new tritium fuel from neutron bombardment — the breeding ratio must exceed 1.0 or the reactor runs out of fuel, since natural tritium is virtually nonexistent

*  Plasma Disruption Frequency: The rate of sudden violent instabilities where confined plasma dumps its enormous thermal energy onto reactor walls in milliseconds — each disruption can severely damage first-wall components and requires repair downtime

*  Divertor Heat Load: The thermal power density on the exhaust component that handles plasma particles escaping confinement — divertor surfaces face heat loads comparable to spacecraft reentry, and no material can survive these conditions indefinitely

My Extension:

New component I want to add: ________________________________

How does it connect? (Draw and explain)

What evidence supports these connections?

Reflection Questions

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer after completing all activities.

How does your model illustrate why the Lawson criterion makes fusion a fundamentally harder engineering challenge than fission?

Why does the energy-output-versus-wall-damage trade-off represent an inherent design tension rather than a solvable problem?

What does the confinement collapse scenario reveal about the stability margins of magnetically confined plasma?

How would your model predictions change if a material were discovered that was completely resistant to neutron damage?

What are the limitations of modeling fusion with seven components when a real reactor has thousands of interacting subsystems?

STEM CHALLENGE: Design a Next-Generation Fusion Reactor Approach

| Name: _______________________________ | Date: _______________ |
| --- | --- |

YOUR MISSION

Select the most critical engineering bottleneck preventing commercial fusion and design an innovative approach to overcome it, using evidence from your computational model to justify your strategy.

THE SCENARIO

The Department of Energy has launched a grand challenge for student teams to propose breakthrough approaches to commercial fusion energy. Your team must identify which variable in the fusion system represents the most critical bottleneck — plasma stability, neutron-resistant materials, tritium fuel supply, or energy balance optimization — and propose an engineering concept that could transform that variable, backed by quantitative evidence from your model.

ENGINEERING DESIGN THINKING

What type of magnetic confinement does your design use (tokamak, stellarator, or alternative) and what advantage does it provide?

How does your design address the neutron damage problem over a 30-year operational lifetime?

What Q factor does your design target and what evidence from your model supports its achievability?

What is the estimated cost and timeline for your concept to reach a net-energy-producing prototype?

MY DESIGN

Draw your design here:

EXPLAIN YOUR DESIGN

Why did you design it this way? Use scientific reasoning!