ModelIt!

Student Activity Pack

Can We Capture Carbon from Thin Air?

Modeling Direct Air Capture Systems, Energy Requirements, and Scalability

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Grade Level: 11th Grade — Level 3: Emerging Science & Technology  |  Standard: HS-ESS3-4, HS-PS1-4, HS-ETS1-3

What's Inside

| Name: _______________________________ | Date: _______________ |
| --- | --- |

1. Pre-Assessment  -  What do you already know?

2. Scoring Rubric  -  How your work will be assessed

3. Component Analysis  -  Sort and classify system parts

4. Model Recording Page  -  Sketch your model

5. Simulation Observations  -  Record what happens

6. Research & Extend  -  Add new components

7. Reflection Questions  -  What did you learn?

8. STEM Challenge  -  Design a Regional DAC Deployment Strategy

Pre-Assessment: What Do You Know?

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer these questions BEFORE the lesson begins.

What is direct air capture and how is it different from planting trees to absorb CO2?

Why do you think removing CO2 from the atmosphere is harder than preventing CO2 emissions in the first place?

Draw a diagram showing how you think a machine could pull CO2 out of regular air and what happens to the CO2 afterward.

If we could build machines to remove CO2, should we still worry about reducing emissions? Why or why not?

Scoring Rubric

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Use this rubric to check your own work!

| Skill | 4 - Expert | 3 - Proficient | 2 - Developing | 1 - Beginning |
| --- | --- | --- | --- | --- |
| Identifying Components | All correct + explains role | Most correct | Some correct | Struggles to identify |
| Establishing Relationships | All arrows correct + explains | Most correct | Some correct | Incorrect |
| Running Simulations | Tests all; predicts outcomes | Runs all; records | Runs some with guidance | Needs help |
| Explaining Results | Clear with evidence | Good explanation | Partial | Struggles |

Component Analysis

| Name: _______________________________ | Date: _______________ |
| --- | --- |

What is a Component?

A component is a part of a system. In our Can We Capture Carbon from Thin Air? system, we have these components:

*  Energy Source Carbon Intensity: The CO2 emissions per unit of energy used to power the DAC system — if the energy comes from fossil fuels, the capture process itself emits CO2, potentially negating the removal benefit

*  Sorbent Efficiency: The amount of CO2 captured per unit of sorbent material per cycle — affected by sorbent chemistry, air contact time, temperature, and humidity conditions

*  Thermal Energy Requirement: The heat energy needed to regenerate the sorbent by releasing captured CO2 — typically 80-120 degrees Celsius for solid sorbents, consuming 1,500-2,500 kWh per tonne of CO2 in current systems

*  Net Carbon Removal: The actual CO2 removed from the atmosphere after subtracting all emissions from energy use, construction, sorbent manufacturing, and transportation — must be strongly positive for the system to be useful

*  Capture Cost: The total economic cost per tonne of CO2 removed — currently $400-$1,000 per tonne for operational DAC plants, which must decrease dramatically for gigatonne-scale deployment

*  Deployment Scale: The number and size of DAC installations needed to achieve meaningful climate impact — removing just 1% of annual emissions would require thousands of large facilities

Sorting Activity

Sort into EXTERNAL (inputs from outside) and INTERNAL (processes inside the system):

| EXTERNAL Components (Come from outside) | INTERNAL Components (Happen inside system) |
| --- | --- |

Explain your reasoning:

Model Recording Page

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Sketch your model showing all components and their relationships.

Relationship Key:

(+) POSITIVE: When one increases, the other ALSO increases

(-) NEGATIVE: When one increases, the other DECREASES

My Model Sketch:

Draw components as boxes. Connect with arrows labeled + or -.

Explain ONE relationship in your model. Why does it work that way?

Simulation Observations

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Run each scenario in ModelIt! and record your observations.

Renewable-Powered DAC

Settings: 100% renewable energy, current best sorbent technology

What do you predict the cost per tonne and energy requirement will be for a purely renewable-powered DAC system?

Observation: What actually happened? Describe what you saw on the graph.

Fossil-Powered DAC

Settings: Natural gas powered capture system

What do you predict the net carbon removal will be when the capture process itself uses fossil energy?

Observation: What actually happened? Describe what you saw on the graph.

Gigatonne Scale Projection

Settings: Scale needed to remove 1 billion tonnes of CO2 per year

What do you predict the total energy demand looks like when you scale DAC to climate-relevant levels?

Observation: What actually happened? Describe what you saw on the graph.

Research & Extend

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Real systems are more complex. EXTEND your model with new components!

Possible New Components:

*  Water Consumption: The amount of water used in the DAC process — some sorbent systems require significant water, creating competition with agricultural and municipal water supplies in arid regions where solar energy is abundant

*  Sorbent Lifetime: The number of capture-release cycles a sorbent material can undergo before it degrades and must be replaced — longer-lasting sorbents reduce operational costs and waste but may have lower per-cycle efficiency

*  Storage Permanence: The guaranteed duration of CO2 storage — geological mineralization is essentially permanent, depleted oil reservoirs last millennia, and enhanced weathering depends on mineral type — permanence determines whether the removal is real or temporary

My Extension:

New component I want to add: ________________________________

How does it connect? (Draw and explain)

What evidence supports these connections?

Reflection Questions

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer after completing all activities.

How does your model demonstrate why the energy source is the most critical variable for whether DAC actually removes carbon from the atmosphere?

What does the concentration difference between atmospheric CO2 (420 ppm) and smokestack CO2 (120,000 ppm) tell you about why DAC is so energy-intensive?

Why is the moral hazard argument — that DAC reduces urgency for emission reductions — a legitimate policy concern rather than just a philosophical objection?

How would your model change if a revolutionary sorbent were discovered that captured CO2 at ten times the efficiency of current materials?

What are the limitations of modeling DAC deployment without including competing uses for renewable energy like replacing fossil fuel electricity and electrifying transportation?

STEM CHALLENGE: Design a Regional DAC Deployment Strategy

| Name: _______________________________ | Date: _______________ |
| --- | --- |

YOUR MISSION

Design a 10-year deployment plan for direct air capture in your region, specifying energy source, site selection, storage method, cost reduction pathway, and the tonnes of CO2 to be removed annually.

THE SCENARIO

Your state government has committed to achieving net-zero emissions by 2040 and has identified direct air capture as one component of the strategy. Your team must design a DAC deployment plan that specifies where to build facilities, what energy source to use, where to store the captured CO2, how costs will decrease over the 10-year period, and how many tonnes per year the system will remove. The plan must also address whether DAC investment could be better spent on other climate strategies.

ENGINEERING DESIGN THINKING

What specific DAC technology would you deploy and what is its current energy requirement per tonne of CO2?

How would you reduce costs from $600/tonne to below $200/tonne over your 10-year deployment plan?

What is the total renewable energy capacity needed to power your DAC facilities, and how does this compare to your region's current renewable capacity?

How would you address the moral hazard argument — that DAC investment reduces urgency for emission reductions?

MY DESIGN

Draw your design here:

EXPLAIN YOUR DESIGN

Why did you design it this way? Use scientific reasoning!