ModelIt!

Student Activity Pack

How Do Self-Driving Cars See?

Modeling LiDAR, Computer Vision, and Autonomous Decision-Making

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Grade Level: 11th Grade — Level 3: Emerging Science & Technology  |  Standard: HS-PS4-1, HS-ETS1-2

What's Inside

| Name: _______________________________ | Date: _______________ |
| --- | --- |

1. Pre-Assessment  -  What do you already know?

2. Scoring Rubric  -  How your work will be assessed

3. Component Analysis  -  Sort and classify system parts

4. Model Recording Page  -  Sketch your model

5. Simulation Observations  -  Record what happens

6. Research & Extend  -  Add new components

7. Reflection Questions  -  What did you learn?

8. STEM Challenge  -  Design an Autonomous Vehicle Sensor Suite

Pre-Assessment: What Do You Know?

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer these questions BEFORE the lesson begins.

What do you already know about how self-driving cars detect objects around them, and what sensors do you think they use?

Why might a self-driving car have difficulty in heavy rain or fog when a human driver can still navigate safely?

Draw a diagram showing how you think a self-driving car builds a picture of its surroundings using different types of sensors.

What is the difference between a car that detects an obstacle and a car that decides what to do about it?

Scoring Rubric

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Use this rubric to check your own work!

| Skill | 4 - Expert | 3 - Proficient | 2 - Developing | 1 - Beginning |
| --- | --- | --- | --- | --- |
| Identifying Components | All correct + explains role | Most correct | Some correct | Struggles to identify |
| Establishing Relationships | All arrows correct + explains | Most correct | Some correct | Incorrect |
| Running Simulations | Tests all; predicts outcomes | Runs all; records | Runs some with guidance | Needs help |
| Explaining Results | Clear with evidence | Good explanation | Partial | Struggles |

Component Analysis

| Name: _______________________________ | Date: _______________ |
| --- | --- |

What is a Component?

A component is a part of a system. In our How Do Self-Driving Cars See? system, we have these components:

*  LiDAR Resolution: The density of laser points per square meter at a given distance — higher resolution creates more detailed 3D maps but generates exponentially more data that must be processed in real time

*  Camera Detection Accuracy: The percentage of objects correctly identified and classified by the computer vision neural network — affected by lighting conditions, object novelty, occlusion, and the diversity of the training dataset

*  Sensor Fusion Confidence: The system's calculated probability that its unified environmental model is correct — increases when multiple sensors agree on object location and classification, decreases when sensors provide conflicting data

*  Processing Latency: The time required to collect sensor data, fuse it into a coherent model, classify all objects, plan a safe path, and send commands to steering, braking, and acceleration actuators

*  Environmental Degradation: The reduction in sensor performance caused by weather, lighting, and road conditions — rain scatters LiDAR beams, fog reduces camera range, sun glare blinds cameras, and snow obscures lane markings and road edges

*  Decision Algorithm Reliability: The probability that the path-planning algorithm selects the safest action given the perceived environment — affected by the completeness of the environmental model, the novelty of the scenario, and whether the situation was represented in training data

*  Safety Margin: The buffer between the vehicle's current trajectory and the nearest potential collision — measured in both distance and time, this margin must account for sensor uncertainty, processing latency, and the unpredictable behavior of other road users

Sorting Activity

Sort into EXTERNAL (inputs from outside) and INTERNAL (processes inside the system):

| EXTERNAL Components (Come from outside) | INTERNAL Components (Happen inside system) |
| --- | --- |

Explain your reasoning:

Model Recording Page

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Sketch your model showing all components and their relationships.

Relationship Key:

(+) POSITIVE: When one increases, the other ALSO increases

(-) NEGATIVE: When one increases, the other DECREASES

My Model Sketch:

Draw components as boxes. Connect with arrows labeled + or -.

Explain ONE relationship in your model. Why does it work that way?

Simulation Observations

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Run each scenario in ModelIt! and record your observations.

Clear Highway Driving

Settings: Maximum LiDAR, minimal environmental degradation, increasing speed

What do you predict happens to the safety margin as vehicle speed doubles while processing latency remains constant?

Observation: What actually happened? Describe what you saw on the graph.

Heavy Rain Urban Intersection

Settings: 40% LiDAR degradation, 30% camera degradation, complex urban scene

What do you predict happens to sensor fusion confidence when the two primary sensors both lose significant capability simultaneously?

Observation: What actually happened? Describe what you saw on the graph.

Novel Object Emergency

Settings: Unclassified obstacle appears in the vehicle's path at highway speed

What do you predict the decision algorithm does when it detects something in the road but cannot determine what it is?

Observation: What actually happened? Describe what you saw on the graph.

Research & Extend

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Real systems are more complex. EXTEND your model with new components!

Possible New Components:

*  V2X Communication: Vehicle-to-everything communication allows autonomous vehicles to receive data from traffic signals, other vehicles, and road infrastructure — extending perception beyond line-of-sight sensors to include information about conditions around corners, over hills, and at distant intersections

*  Prediction Horizon: The time window over which the vehicle's algorithms predict the future positions of all detected moving objects — longer prediction horizons enable earlier and smoother responses but become exponentially less accurate as behavioral uncertainty compounds

*  Ethical Decision Framework: The programmed hierarchy of values the vehicle uses when a collision is unavoidable — how it prioritizes between protecting its passengers, protecting pedestrians, minimizing total harm, and following traffic laws when these objectives conflict

My Extension:

New component I want to add: ________________________________

How does it connect? (Draw and explain)

What evidence supports these connections?

Reflection Questions

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer after completing all activities.

How does your model demonstrate the fundamental trade-off between sensor resolution and processing latency?

Why is sensor fusion more valuable than simply using the best single sensor, and what happens when sensors provide contradictory information?

What does your model reveal about why autonomous vehicles perform differently in clear conditions versus adverse weather?

How would your model change if processing latency could be reduced to near-zero through advanced computing hardware?

What are the limitations of modeling autonomous vehicle safety when real-world driving includes situations that have never occurred before?

STEM CHALLENGE: Design an Autonomous Vehicle Sensor Suite

| Name: _______________________________ | Date: _______________ |
| --- | --- |

YOUR MISSION

Design an optimal sensor configuration for an autonomous vehicle that must operate safely in your city's specific driving conditions, addressing sensor selection, placement, fusion architecture, and failure mode handling.

THE SCENARIO

A transportation company is deploying autonomous delivery vehicles in your city and has hired your team to design the sensor suite and perception system. Your city has specific challenges — weather patterns, road conditions, pedestrian density, and infrastructure — that the sensor system must handle. You must select and position sensors, design the fusion architecture, establish minimum safety margins for different conditions, and create protocols for what the vehicle does when sensor confidence drops below safe thresholds.

ENGINEERING DESIGN THINKING

How many and what type of LiDAR units would you mount on the vehicle and where would you position them to minimize blind spots?

What camera specifications (resolution, frame rate, field of view, low-light sensitivity) are required for reliable object classification at highway speeds?

What is your maximum acceptable processing latency and how does this constrain your computing hardware requirements?

What failure cascade protocol would you implement when environmental conditions degrade sensor confidence below your safety threshold?

MY DESIGN

Draw your design here:

EXPLAIN YOUR DESIGN

Why did you design it this way? Use scientific reasoning!